Servo Motor Control Using 8051 Micrcontroller

The control signals for the rotation of the motor are provided by 8051 microcontroller.For basic concepts and know-how of a servo motor, refer to the article Servo Motor.

Source Code

Here,i have used AT89S51  microcontroller.Code is written in C using  Keil uVision4.

#include<reg51.h>

sbit output=P1^0;          //Output to motor

void timer(int msec)     // Function for timer

{

int i;

for(i=0;i<msec;i++)

{

TMOD=0x20;         // Mode2

TH1=0xFF;

TL1=0xD1;

//TH1= -23;        // 50usec timer

TR1=1;

while(TF1==0);

TF1=0;

TR1=0;

}

}

void main()

{

int i;

output=0;

while(1)

{

//anticlockwise direction

for(i=0;i<500;i++)

{

output=1;

timer(20);

output=0;

timer(360);

}

//clockwise direction

for(i=0;i<500;i++)

{

output=1;

timer(40);

output=0;

timer(360);

}

}

}